TMC Bench Crane — Assembly Guide

Interactive assembly procedure & parts list

CONFIDENTIAL — Trimble internal use only. Do not share or distribute.

| Product | TMC Bench Wire Crane |
| --- | --- |
| Document type | Assembly guide export |
| Generated | 2026-08-13 |
| Interactive source | TMC Bench Crane guide-data.js |

This document was exported from the interactive assembly guide. Key step reference photos are embedded for offline use.

360° 3D model (Fusion): https://a360.co/4en8lcl

Overview

How this guide is organized

The TMC bench crane assembly is broken into separate segments. Each segment matches a folder in the STL library and has its own print checklist, hardware list, and step-by-step assembly instructions.

Start here for required tools and the master shopping list. Then work through each segment in order — hardware called out in a segment is only what you need for that part of the build.

This guide is still being expanded. Segments marked “Coming soon” are not ready yet; hardware totals will change as those sections are finished.

Required tools

Have these ready before you start printing or assembling.

2.5 mm hex key (Allen)

T-handle may help — especially on repeated M3 fasteners

Used for: M3×8 cap screws throughout the crane (socket head uses 2.5 mm hex — not 3 mm)

5 mm hex key (Allen)

T-handle may help — especially on M6 fasteners in tight spaces

Used for: M6×20 and M6×30 cap screws — base plates, boom mounts, VRU, extrusions (socket head uses 5 mm hex — not 6 mm)

Soldering iron

Used for: Heating ruthex® RX-M3×5.7 and RX-M6Sx6.8 inserts into printed parts

Required for installing ruthex® heat-set inserts. Use a soldering iron compatible with 900M- and T18-series tips (see ruthex tip set below).

Note: Do not exceed 300°C when installing inserts

ruthex® heat-set tip set

Use the tip that matches each insert size: M3 tip for RX-M3×5.7 inserts, M6 tip for RX-M6Sx6.8 inserts. Tips are compatible with 900M- and T18-series soldering irons.

Note: Tips are for installing inserts only — not suitable for removal

Note: Pair with a soldering iron before starting assembly

Reference photo

ruthex heat-set insert tip set — M2 through M8 tips for 900M and T18 soldering irons

https://drive.google.com/file/d/1es6bZtfu68YrNKheIwu9nM0c3LW4wfZr/view

Full crane hardware & parts

Master list status

INCOMPLETE — This master list reflects what is documented so far (Base Plates, partial Boom, and Bucket Attachment hardware). Totals will be updated as remaining segments are added. Do not use this as a final BOM until the guide is complete.

This crane reuses a small set of Ruthex inserts, cap screws, and dowels across all segments. Quantities below will be finalized as each segment is completed.

| Qty | Part / specification |
| --- | --- |
|  | ruthex® RX-M3×5.7 Threaded Insert — P/N GE-M3X5.7-001 — Primary insert — used throughout boom assembly |
|  | ruthex® RX-M6Sx6.8 Threaded Insert — P/N GE-M6X68-001 — Boom mounts only — order 8 for VRU flexibility, 6 minimum |
|  | M3 × 8 mm Socket Head Cap Screw — Single M3 screw size for entire build |
|  | M6 × 20 mm Socket Head Cap Screw — 14× total — 8 base · 4 boom mount · 2 VRU |
|  | M6 × 30 mm Socket Head Cap Screw — 2× — LR970B boom IMU mount |
|  | M3 × 12 mm Steel Dowel Pin — Single dowel size for entire build |
|  | 5 mm ID × 14 mm OD × 5 mm Ball Bearing — 4× for dual sheave · 2× for single sheave |
|  | 5 mm × 150 mm Sheave Pin — 2× for dual sheave boom head |
|  | 10 mm × 150 mm Boom Pivot Pin — 1× main boom pivot |
|  | 40 mm × 40 mm Lite T-Slotted Profile — 500 mm — P/N 40-4040-LITE-BLACK — 2× 500 mm rails — P/N 40-4040-LITE-BLACK |
|  | M6 Short Self-Aligning Roll-in T-Nut with Spring Leaf — P/N 13093 — 8× — P/N 13093 · base plate to extrusion |
|  | 40 Series End Cap with Molded Push-In Stem — P/N 12260 — 4× — P/N 12260 · extrusion rail ends |

Hardware catalog reference

| Part | P/N / spec | Used in |
| --- | --- | --- |
| ruthex® RX-M3×5.7 Threaded Insert | P/N GE-M3X5.7-001 · M3 · 5.7 mm · hole 4.0 mm diameter | Boom lattice, lower boom support, shaft retainers, upper clamp mounts, bucket attachment |
| ruthex® RX-M6Sx6.8 Threaded Insert | P/N GE-M6X68-001 · M6 · 6.8 mm · hole 8.0 mm diameter | Boom mounts (VRU) · LR970B boom IMU mount |
| M3 × 8 mm Socket Head Cap Screw |  | Boom lattice joints, pulley halves, shaft retainers, lower boom support, bucket attachment |
| M6 × 20 mm Socket Head Cap Screw |  | Base plates to extrusions (8) · boom mounts to base plate (4) · VRU to boom mount (2) |
| M6 × 30 mm Socket Head Cap Screw |  | LR970B boom IMU mount — 2× into RX-M6Sx6.8 inserts |
| M3 × 12 mm Steel Dowel Pin |  | Base plate joint (optional), boom lattice section alignment, bucket attachment |
| 5 mm ID × 14 mm OD × 5 mm Ball Bearing |  | Sheave (pulley) assembly — one per pulley; bucket attachment pivot |
| 5 mm × 150 mm Sheave Pin |  | Boom head — sheaves slide on these pins; bucket attachment pivot |
| 10 mm × 150 mm Boom Pivot Pin |  | Lower boom support — main boom pivot |
| 40 mm × 40 mm Lite T-Slotted Profile — 500 mm | P/N 40-4040-LITE-BLACK · 500 mm (19.69 in) | Base plate foundation rails — qty 2 |
| M6 Short Self-Aligning Roll-in T-Nut with Spring Leaf | P/N 13093 · M6 | Base plates to extrusion — one per M6×20 screw · qty 8 |
| 40 Series End Cap with Molded Push-In Stem | P/N 12260 | Extrusion rail ends — qty 4 (both ends of both rails) |

Assembly segments (in order)

Assembly segments — open any section for that segment’s hardware, prints, and instructions.

| # | Segment |
| --- | --- |
| 1 | Axio Group — In progress |
| 2 | Base Plates — Ready |
| 3 | Boom — In progress |
| 4 | Boom Spacers for Dual Sheave — Coming soon |
| 5 | Boom Supports — Coming soon |
| 6 | Multi Turn Add On — Coming soon |
| 7 | Bucket Attachment — In progress |

Axio Group

Segment status: In progress

STL library folder: AXIO GROUP

Assemble the Axio Group here first, then install it on the base plates in Base Plates · Step 3.

Step 1: Assemble Axio Group (REQUIRED)

Assemble the Axio Group per the AXIO GROUP prints. Detailed step-by-step photos and hardware lists are being added — check back as this segment is expanded.

Warnings

Assembly instructions for printed parts and internal hardware are still being documented.

Checks before continuing:

Axio Group fully assembled per AXIO GROUP prints

Ready to mount on base plate assembly in Base Plates · Step 3

Reference photo

Axio Group assembly — photos coming soon

Base Plates

Segment status: Ready

STL library folder: BASE PLATES

Extrusions are always the last step. Complete plate join, boom mounts, and Axio Group mount (Step 3) before extrusions.

STOP — Read before mounting extrusions

Do NOT bolt the base plate assembly to the aluminum extrusions until you have joined the base plates, installed the boom mounts, and installed the Axio Group.

Six M6×20 cap screws must be tightened from the UNDERSIDE of the base plate before extrusions go on: 4× for boom mounts (Step 2) and 2× for the Axio Group (Step 3).

Once the extrusions are installed, you cannot reach those underside screws — the build will be blocked.

Prerequisites before extrusions:

Front & rear base plates joined with 3× M3×8 (Step 1)

Optional M3×12 alignment dowels at plate joint if used (Step 1)

Boom pin installed through boom mounts + lower boom section (Step 2)

Boom mounts bolted from underside — 4× M6×20 (Step 2)

Axio Group assembled (Axio Group segment)

Assembled Axio Group installed — 6× M3×8 + 2× M6×20 (Step 3)

Segment hardware summary

| Qty | Part / specification |
| --- | --- |
| 3 | ruthex® RX-M3×5.7 Threaded Insert — P/N GE-M3X5.7-001 — qty 3 — Rear base plate |
| 9 | M3 × 8 mm Socket Head Cap Screw — qty 9 — 3× join plates + 6× Axio Group mount |
| 1 | 10 mm × 150 mm Boom Pivot Pin — qty 1 — Boom pivot |
| 14 | M6 × 20 mm Socket Head Cap Screw — qty 14 — 4× boom mounts + 2× Axio Group + 8× extrusions |
| 3 | M3 × 12 mm Steel Dowel Pin — qty 3 — Optional plate alignment |
| 2 | 40 mm × 40 mm Lite T-Slotted Profile — 500 mm — P/N 40-4040-LITE-BLACK — qty 2 — 40 Series (metric) · compatible with all 40 series fasteners · order cut to 500 mm length. |
| 8 | M6 Short Self-Aligning Roll-in T-Nut with Spring Leaf — P/N 13093 — qty 8 — Roll into T-slot before inserting cap screw. Replaces generic hammer nuts / inserts. |
| 4 | 40 Series End Cap with Molded Push-In Stem — P/N 12260 — qty 4 — Push-in stem · finishes open extrusion ends. |

Print checklist

| Part name | STL file | Qty | Notes |
| --- | --- | --- | --- |
| Combined Base Plate — Front | COMBINED BASE PLATE FRONT.stl | 1 | BASE PLATES |
| Combined Base Plate — Rear | COMBINED BASE PLATE REAR.stl | 1 | BASE PLATES · 3× RX-M3×5.7 inserts |
| Boom Mount | BOOM MOUNT.stl | 2 | BOOM · Installed in Step 2 — before extrusions |
| Boom End — Lower | BOOM END LOWER.stl | 1 | BOOM · On pivot pin in Step 2 |

Step 1: Join front & rear base plates (REQUIRED)

Install 3× ruthex® RX-M3×5.7 heat-set inserts in the rear base plate, then connect the front and rear combined base plates using 3× M3×8 cap screws. Keep the assembly off the extrusions — the extrusion mount is always the last step.

Procedure:

Install all 3 inserts in the REAR base plate before mating the front plate

Use 4.0 mm holes for RX-M3×5.7 inserts (P/N GE-M3X5.7-001)

Align front plate to rear plate and install 3× M3×8 cap screws into the inserts

Reference legend:

Blue: M3×8 cap screws — join front & rear plates (×3)

Green: M3×12 dowel pins — optional alignment aid (×3)

Printed parts

| Part name | STL file | Qty | Notes |
| --- | --- | --- | --- |
| Combined Base Plate — Front | COMBINED BASE PLATE FRONT.stl | 1 |  |
| Combined Base Plate — Rear | COMBINED BASE PLATE REAR.stl | 1 | 3× RX-M3×5.7 inserts install here |

Hardware

| Qty | Part / specification |
| --- | --- |
| 3 | ruthex® RX-M3×5.7 Threaded Insert — P/N GE-M3X5.7-001 — qty 3 — Rear base plate only |
| 3 | M3 × 8 mm Socket Head Cap Screw — qty 3 — Front plate into rear plate inserts |

Checks before continuing:

All 3× RX-M3×5.7 inserts flush in rear base plate

Front and rear plates aligned and seated together

All 3× M3×8 cap screws installed — plate joint secure

Alignment dowels (not required)

Optionally install 3× M3×12 steel dowel pins at the plate joint before or while mating the front plate. Dowels are not required — they can aid alignment and add rigidity beyond the M3×8 screw connection.

Procedure:

Press dowels into the rear plate dowel holes before mating the front plate

The joint is secured by the 3× M3×8 cap screws — dowels are an alignment aid only

| Qty | Part / specification |
| --- | --- |
| 3 | M3 × 12 mm Steel Dowel Pin — qty 3 — Optional — press-fit at plate joint |

Checks:

All 3 dowels fully seated (if used)

Plate joint solid — no gap at seam

Reference photo

Join base plates — M3×8 cap screws (blue) and optional M3×12 dowel pins (green)

https://drive.google.com/file/d/1eHzC3z76X4utOGR7O3i42zuB1WMiqhbL/view

Step 2: Boom pin, boom mounts & lower boom — bolt from underneath (REQUIRED)

On the joined base plate assembly (still OFF the extrusions), install the 10 mm boom pin through both boom mounts and the lower boom section. Once the pin is fully through, bolt both boom mounts to the base plate from underneath with 4× M6×20 cap screws.

Warnings

REQUIRED before extrusions. These 4 screws are 4 of 6 underside M6×20 cap screws — the extrusion rails will cover this access.

Procedure:

Work with base plates on a flat surface — not yet on extrusions

1. Slide 10 mm × 150 mm boom pin through both boom mounts and the lower boom section

2. Confirm pin is fully through mounts and lower boom section

3. Bolt both boom mounts to base plate from underneath — 4× M6×20 cap screws (blue arrows)

Reference legend:

Blue: Top arrows — boom pin through mounts & lower boom

Blue: Bottom arrows — M6×20 mount screws from underside (×4)

Printed parts

| Part name | STL file | Qty | Notes |
| --- | --- | --- | --- |
| Combined Base Plate — Front | COMBINED BASE PLATE FRONT.stl | 1 | Not on extrusions yet |
| Combined Base Plate — Rear | COMBINED BASE PLATE REAR.stl | 1 |  |
| Boom Mount | BOOM MOUNT.stl | 2 | BOOM |
| Boom End — Lower | BOOM END LOWER.stl | 1 | BOOM · Lower boom section on pivot pin |

Hardware

| Qty | Part / specification |
| --- | --- |
| 1 | 10 mm × 150 mm Boom Pivot Pin — qty 1 — Through both boom mounts and lower boom section |
| 4 | M6 × 20 mm Socket Head Cap Screw — qty 4 — From underside of base plate into boom mounts |

Checks before continuing:

Boom pin fully through both boom mounts and lower boom section

Both boom mounts seated on base plate

All 4× M6×20 cap screws installed from underside

Pivot swings freely before proceeding to extrusion mount

Reference photo

Underside view — M6×20 boom mount screws (blue) before extrusions

https://drive.google.com/file/d/1NytNrWaykBqriE_87UFKLq4RFcrmJXJK/view

Perform before base plates are on extrusions

Step 3: Install assembled Axio Group — bolt from underneath (REQUIRED)

On the joined base plate assembly (still OFF the extrusions), mount the assembled Axio Group and secure it with 6× M3×8 cap screws plus 2× M6×20 cap screws from underneath. The 2× M6×20 are 2 of the 6 underside M6×20 screws that become inaccessible once extrusions are on.

Warnings

Assemble the Axio Group first in the Axio Group segment.

REQUIRED before extrusions. The 2× M6×20 screws are installed from the bottom of the base plate — the extrusion rails will cover this access.

Procedure:

Work with base plates on a flat surface — not yet on extrusions

Position the assembled Axio Group on the base plate

Install 6× M3×8 cap screws to secure the Axio Group to the base plate

Bolt the Axio Group from underneath — 2× M6×20 cap screws

Reference legend:

Blue: M3×8 cap screws — Axio Group to base plate (×6)

Blue: M6×20 cap screws — underside into Axio Group (×2)

Hardware

| Qty | Part / specification |
| --- | --- |
| 6 | M3 × 8 mm Socket Head Cap Screw — qty 6 — Secure Axio Group to base plate |
| 2 | M6 × 20 mm Socket Head Cap Screw — qty 2 — From underside of base plate into Axio Group |

Checks before continuing:

Axio Group seated on base plate

All 6× M3×8 cap screws installed

Both M6×20 cap screws installed from underside

Axio Group secure before proceeding to extrusion mount

Reference photo

Axio Group on base plate — 6× M3×8 and 2× underside M6×20 screws

https://drive.google.com/file/d/1P_uveZbafe54jDlvrlchW-tPoUi9u5JE/view

Perform before base plates are on extrusions

Step 4: NOW mount base plate assembly to extrusions (REQUIRED)

Last step — bolt the complete base plate assembly (plates joined, boom mounts secured, Axio Group installed) to two 500 mm 8020 extrusions. If any of the 6 underside M6×20 screws are missing, STOP and go back to Steps 2 and 3.

Warnings

BLOCKED until Steps 1–3 are complete. Never attach extrusions first.

If you already mounted extrusions: remove them, complete Steps 2 and 3 (6× M6×20 underside screws), then remount.

Procedure:

REAR plate (flange end) goes on the left

FRONT plate (U-opening) goes on the right

Roll M6 T-nuts into T-slots at each screw location before installing cap screws

Install end caps on both ends of both extrusion rails

Printed parts

| Part name | STL file | Qty | Notes |
| --- | --- | --- | --- |
| Combined Base Plate — Front | COMBINED BASE PLATE FRONT.stl | 1 |  |
| Combined Base Plate — Rear | COMBINED BASE PLATE REAR.stl | 1 |  |

Hardware

| Qty | Part / specification |
| --- | --- |
| 2 | 40 mm × 40 mm Lite T-Slotted Profile — 500 mm — P/N 40-4040-LITE-BLACK — qty 2 — 40 Series (metric) · compatible with all 40 series fasteners · order cut to 500 mm length. |
| 8 | M6 Short Self-Aligning Roll-in T-Nut with Spring Leaf — P/N 13093 — qty 8 — One per cap screw — roll into T-slot first |
| 8 | M6 × 20 mm Socket Head Cap Screw — qty 8 — 14× total for full build · also used with 8020 T-nuts on base extrusions. |
| 4 | 40 Series End Cap with Molded Push-In Stem — P/N 12260 — qty 4 — Both ends of both rails |

Checks before continuing:

Plates flush on both extrusions

Front/rear orientation correct

All 8× M6 T-nuts seated and 8× M6×20 cap screws tight

All 4 end caps installed on extrusion rail ends

Extrusions parallel

Reference photo

Base plate assembly mounted on 8020 extrusions — rear left, front right

https://drive.google.com/file/d/16mX-ZVtX3UWbRuCr6iHM3m-zjwtUpehC/view

Reference: 8020.net · 40-4040-Lite-Black · 40 Series · cut to 500 mm

Boom

Segment status: In progress

STL library folder: BOOM

Which boom configuration are you building?

Single sheave: 1× 30 mm + 1× 50 mm pulley

Dual sheave (default): 2× 30 mm + 2× 50 mm pulley

VRU placement on boom mounts

Either side (recommended) (default): 8× ruthex® RX-M6Sx6.8 — VRU on either side

One side only: 6× ruthex® RX-M6Sx6.8 — VRU on one side only

Recommended assembly order: First assemble the LH and RH side halves (bottom + mid + top sections joined on each side). Then assemble the top and bottom panels to close the lattice.

1. Right-hand side half

1. Left-hand side half

2. Top panels

2. Bottom panels

Print checklist

| Part name | STL file | Qty | Notes |
| --- | --- | --- | --- |
| Boom Mount | BOOM MOUNT.stl | 2 |  |
| Boom Section 1 — LH | BOOM SECTION 1 LH.stl | 1 |  |
| Boom Section 1 — RH | BOOM SECTION 1 RH.stl | 1 |  |
| Boom Section 1 — Top/Bottom | BOOM SECTION 1 TOP_BOTTOM.stl | 1 |  |
| Boom Mid Section — LH/RH | BOOM MID SECTION LH_RH.stl | 1 |  |
| Boom Mid Section — Top/Bottom | BOOM MID SECTION TOP_BOTTOM.stl | 1 |  |
| Boom End — Lower | BOOM END LOWER.stl | 1 |  |
| Boom End — Upper | BOOM END UPPER.stl | 1 |  |
| Boom Spacer | BOOM SPACER.stl | 1 |  |
| Upper Boom Clamp | UPPER BOOM CLAMP.stl | 4 |  |
| Boom End Dual Sheave — LH | BOOM END DUAL SHEAVE LH.stl | 1 |  |
| Boom End Dual Sheave — RH | BOOM END DUAL SHEAVE RH.stl | 1 |  |
| Boom LR970B — Front Plate | BOOM LR970B FRONT PLATE.stl | 1 |  |
| Boom LR970B — Back Plate | BOOM LR970B BACK PLATE.stl | 1 |  |

Additional prints — single-sheave

| Part name | STL file | Qty | Notes |
| --- | --- | --- | --- |
| 30 mm Pulley Half 1 | 30MM PULLEY (PRIMARY SHEAVE) HALF 1.stl | 1 |  |
| 30 mm Pulley Half 2 | 30MM PULLEY (PRIMARY SHEAVE) HALF 2.stl | 1 |  |
| 50 mm Pulley Half 1 | 50MM PULLEY (SECONDARY SHEAVE) HALF 1.stl | 1 |  |
| 50 mm Pulley Half 2 | 50MM PULLEY (SECONDARY SHEAVE) HALF 2.stl | 1 |  |
| Sheave Spacer (single sheave only) | SHEAVE SPACER.stl | 4 | LH & RH of each sheave to center on boom |

Do not print files from DUAL SHEAVE BOOM SPACER folder

Additional prints — dual-sheave

| Part name | STL file | Qty | Notes |
| --- | --- | --- | --- |
| 30 mm Pulley Half 1 | 30MM PULLEY (PRIMARY SHEAVE) HALF 1.stl | 2 |  |
| 30 mm Pulley Half 2 | 30MM PULLEY (PRIMARY SHEAVE) HALF 2.stl | 2 |  |
| 50 mm Pulley Half 1 | 50MM PULLEY (SECONDARY SHEAVE) HALF 1.stl | 2 |  |
| 50 mm Pulley Half 2 | 50MM PULLEY (SECONDARY SHEAVE) HALF 2.stl | 2 |  |
| Center 8 mm Spacer | CENTER 8MM SPACER.stl | 2 | DUAL SHEAVE BOOM SPACER |
| LH 48 mm Spacer | LH 48MM SPACER.stl | 2 | DUAL SHEAVE BOOM SPACER |
| RH 35 mm Spacer | RH 35MM SPACER.stl | 2 | DUAL SHEAVE BOOM SPACER |

Do not print SHEAVE SPACER.stl (single sheave only)

Step 1: Boom mount & pivot pin (REQUIRED)

Completed in Axio Group segment + Base Plates · Steps 1–4 — assemble Axio Group, join plates, boom mounts, mount Axio, then extrusions.

Applies to configuration:

single-sheave

dual-sheave

Warnings

If not done yet, complete the Axio Group segment and Base Plates · Steps 1–4 in order. Do not mount to extrusions before all 6 underside M6×20 screws are installed (4× boom mounts + 2× Axio Group).

Checks before continuing:

Axio Group segment complete — Axio Group assembled

Base Plates · Step 1 complete — front & rear plates joined

Base Plates · Step 2 complete — pin through mounts & lower boom

Base Plates · Step 3 complete — Axio Group bolted from underside

Base Plates · Step 4 complete — assembly on extrusions

Reference photo

See Axio Group segment + Base Plates — steps 1–4

Step 2: Assemble LH & RH side halves (REQUIRED)

Assemble both side halves first. On each side, join the bottom, mid, and top lattice sections using Ruthex inserts, cap screws, and dowels at the four joint locations (two joints × two fasteners per joint).

Applies to configuration:

single-sheave

dual-sheave

Procedure:

Build the right-hand side half completely before starting the left

Each side = bottom section + mid section + top section

4 joint fasteners per side at bottom↔mid and mid↔top seams (see arrows)

At the top section of each side, install 6× ruthex® RX-M3×5.7 inserts for upper boom clamp mounts

Printed parts

| Part name | STL file | Qty | Notes |
| --- | --- | --- | --- |
| Boom Section 1 — LH | BOOM SECTION 1 LH.stl | 1 | Bottom/mid/top side panels — LH |
| Boom Section 1 — RH | BOOM SECTION 1 RH.stl | 1 | Bottom/mid/top side panels — RH |
| Boom Mid Section — LH/RH | BOOM MID SECTION LH_RH.stl | 1 |  |
| Boom End — Lower | BOOM END LOWER.stl | 1 |  |
| Boom End — Upper | BOOM END UPPER.stl | 1 |  |

Hardware per side

| Qty | Part / specification |
| --- | --- |
| 4 | ruthex® RX-M3×5.7 Threaded Insert — P/N GE-M3X5.7-001 — qty 4 — Section joints — 2 at bottom↔mid + 2 at mid↔top |
| 4 | M3 × 8 mm Socket Head Cap Screw — qty 4 — Into RX-M3×5.7 inserts at section joints |
| 2 | M3 × 12 mm Steel Dowel Pin — qty 2 — Assists alignment at section joints |
| 6 | ruthex® RX-M3×5.7 Threaded Insert — P/N GE-M3X5.7-001 — qty 6 — Top section only — for upper boom clamp mounts |

Checks before continuing:

RH side: bottom, mid, and top sections joined — 4 screws + 2 dowels

LH side: bottom, mid, and top sections joined — 4 screws + 2 dowels

6 clamp-mount inserts installed in top of each side

Both side halves square and rigid at joints

Reference photo

One side half — 4 arrow callouts at bottom↔mid and mid↔top joints

Step 3: Assemble top & bottom panels (REQUIRED)

With both LH and RH side halves complete, assemble the top and bottom panel sections. The top section has a recess in the segment closest to the sheaves — orient toward the boom head. Use the same section-joint hardware as the side halves for joins within each panel run.

Applies to configuration:

single-sheave

dual-sheave

Procedure:

The top boom section has a recess in the segment closest to the sheaves — orient this toward the boom head

Assemble top and bottom panel runs (section-to-section) before final 4-way lattice join

Printed parts

| Part name | STL file | Qty | Notes |
| --- | --- | --- | --- |
| Boom Section 1 — Top/Bottom | BOOM SECTION 1 TOP_BOTTOM.stl | 1 |  |
| Boom Mid Section — Top/Bottom | BOOM MID SECTION TOP_BOTTOM.stl | 1 |  |

Hardware per side

| Qty | Part / specification |
| --- | --- |
| 4 | ruthex® RX-M3×5.7 Threaded Insert — P/N GE-M3X5.7-001 — qty 4 — Per panel run at internal section joints |
| 4 | M3 × 8 mm Socket Head Cap Screw — qty 4 — Into RX-M3×5.7 at internal section joints |
| 2 | M3 × 12 mm Steel Dowel Pin — qty 2 — At internal section joints |

Checks before continuing:

Top panel run assembled — recess faces toward sheaves / boom head

Bottom panel run assembled

Internal section joints secured on both panel runs

Reference photo

Top panel — recess callout at sheave end

Step 4: Join all 4 sides — complete boom lattice (REQUIRED)

Final boom lattice step. Before joining, install Ruthex inserts and dowels on the mating edges of the top and bottom sections. Lay one side half (LH or RH) flat on a table, then mate the remaining three sides to close the lattice.

Applies to configuration:

single-sheave

dual-sheave

Procedure:

Install top/bottom edge prep BEFORE joining the four sides

Blue arrows = ruthex® RX-M3×5.7 insert locations on mating edges

Green arrows = M3×12 mm dowel locations on mating edges

Start by laying LH or RH side half flat on the table

Reference legend:

Blue: ruthex® RX-M3×5.7 (GE-M3X5.7-001)

Green: M3×12 mm dowel locations

Printed parts

| Part name | STL file | Qty | Notes |
| --- | --- | --- | --- |
| Boom Section 1 — LH | BOOM SECTION 1 LH.stl | 1 | Completed side half |
| Boom Section 1 — RH | BOOM SECTION 1 RH.stl | 1 | Completed side half |
| Boom Section 1 — Top/Bottom | BOOM SECTION 1 TOP_BOTTOM.stl | 1 | Completed top/bottom runs |
| Boom Mid Section — Top/Bottom | BOOM MID SECTION TOP_BOTTOM.stl | 1 |  |

Hardware totals

Prep — top & bottom mating edges (before join)

| Qty | Part / specification |
| --- | --- |
| 28 | ruthex® RX-M3×5.7 Threaded Insert — P/N GE-M3X5.7-001 — qty 28 — Blue arrow locations — both top & bottom |
| 24 | M3 × 12 mm Steel Dowel Pin — qty 24 — Green arrow locations — both top & bottom |

Final 4-way assembly

| Qty | Part / specification |
| --- | --- |
| 12 | M3 × 12 mm Steel Dowel Pin — qty 12 — Optional — assists joining all 4 sides |

Checks before continuing:

28× ruthex® RX-M3×5.7 inserts installed on top & bottom mating edges

24 M3×12 dowels installed on top & bottom (green locations)

One side half laid flat as base — other three sides mated

12 optional dowels installed if used for final alignment

Full lattice closed, square, and rigid

Reference photo

Mating edge — blue = Ruthex inserts, green = dowels

Step 5: Assemble sheaves (pulleys) (REQUIRED)

Join each pulley half-pair, press in bearing, install 3× M3×8 cap screws per pulley (direct thread — no inserts).

Applies to configuration:

single-sheave

dual-sheave

Hardware totals

Items

bearings: 2

M3×8 screws: 6

pulleys: 2

Items

bearings: 4

M3×8 screws: 12

pulleys: 4

Checks before continuing:

Bearing fully seated in center hub

All 3 M3 screws installed per pulley

Pulley spins freely on bearing

Reference photo

Pulley halves + bearing + M3 screw callouts

Step 6: Install pins, sheaves & shaft retainers (REQUIRED)

With the boom lattice complete, slide two 150 mm × 5 mm pins through the sheave locations. Add dual sheave spacers as each pin goes in to position the sheaves. Bolt the four shaft retainers (upper boom clamps) with 12× M3×8 cap screws into the Ruthex inserts (yellow arrows).

Applies to configuration: dual-sheave

Procedure:

Pre-assemble sheaves in Step 5 before installing on pins

Install spacers on the pin before each sheave — do not slide all sheaves on first

Two pins = two sheave rows (upper and lower shaft locations)

Reference legend:

Yellow: M3×8 cap screw — shaft retainer (×12 total, 3 per retainer)

Blue: ruthex® RX-M3×5.7 — retainer mount

Green: Dual sheave spacers on pin

Printed parts

| Part name | STL file | Qty | Notes |
| --- | --- | --- | --- |
| Upper Boom Clamp (shaft retainer) | UPPER BOOM CLAMP.stl | 4 |  |
| Center 8 mm Spacer | CENTER 8MM SPACER.stl | 2 |  |
| LH 48 mm Spacer | LH 48MM SPACER.stl | 2 |  |
| RH 35 mm Spacer | RH 35MM SPACER.stl | 2 |  |

Hardware

| Qty | Part / specification |
| --- | --- |
| 2 | 5 mm × 150 mm Sheave Pin — qty 2 — Slide through sheave locations |
| 12 | M3 × 8 mm Socket Head Cap Screw — qty 12 — Shaft retainers — yellow arrows, into RX-M3×5.7 inserts |

Checks before continuing:

Both 150 mm × 5 mm pins fully inserted

Spacers installed between sheaves and frame as each pin was slid through

All 4 sheaves seated and spin freely on pins

All 4 shaft retainers bolted — 12× M3×8 at yellow arrow locations

Pins captured — retainers prevent pin from sliding out

Reference photo

Boom head — yellow = retainer screws, green = spacers on pins

Step 7: Install lower boom support (REQUIRED)

Install 8× ruthex® RX-M3×5.7 heat-set inserts in the lower boom support and bolt it to the lower boom lattice with 8× M3×8 cap screws. The 10 mm × 150 mm boom pin was installed through mounts and lower boom in Base Plates · Step 2 — verify it is still seated.

Applies to configuration:

single-sheave

dual-sheave

Reference legend: Blue: ruthex® RX-M3×5.7 (×8) — P/N GE-M3X5.7-001

Printed parts

| Part name | STL file | Qty | Notes |
| --- | --- | --- | --- |
| Boom End — Lower | BOOM END LOWER.stl | 1 | Already on boom pin from Base Plates · Step 2 |

Hardware

| Qty | Part / specification |
| --- | --- |
| 8 | ruthex® RX-M3×5.7 Threaded Insert — P/N GE-M3X5.7-001 — qty 8 — Install before bolting to lattice |
| 8 | M3 × 8 mm Socket Head Cap Screw — qty 8 — Bolt lower boom support to lower boom lattice |

Checks before continuing:

All 8× ruthex® RX-M3×5.7 inserts installed flush in lower boom support

Lower boom support bolted securely to lower boom lattice

10 mm × 150 mm boom pin still fully seated through mounts and lower boom

Pivot operates smoothly

Reference photo

Lower boom support — blue arrows = Ruthex insert locations

Step 8: Install ruthex® RX-M6Sx6.8 in boom mounts (REQUIRED)

Install ruthex® RX-M6Sx6.8 heat-set inserts into both boom mounts for VRU mounting. Use 8 inserts if you want the option to mount the VRU on either side of the boom; 6 if VRU will only ever mount on one side.

Applies to configuration:

single-sheave

dual-sheave

Procedure:

Install inserts before attaching VRU to boom mounts

Blue arrows show insert hole locations on each boom mount

Use 8.0 mm diameter holes for RX-M6Sx6.8 inserts

Reference legend: Blue: ruthex® RX-M6Sx6.8 (GE-M6X68-001)

Printed parts

| Part name | STL file | Qty | Notes |
| --- | --- | --- | --- |
| Boom Mount | BOOM MOUNT.stl | 2 |  |

Hardware (VRU configuration dependent)

vru-both-sides

| Qty | Part / specification |
| --- | --- |
| 8 | ruthex® RX-M6Sx6.8 Threaded Insert — P/N GE-M6X68-001 — qty 8 — Both mounts fully prepped — VRU can mount either side |

vru-one-side

| Qty | Part / specification |
| --- | --- |
| 6 | ruthex® RX-M6Sx6.8 Threaded Insert — P/N GE-M6X68-001 — qty 6 — One side only — see blue arrow locations |

Checks before continuing:

All ruthex® RX-M6Sx6.8 inserts installed flush in boom mounts

Insert count matches VRU placement choice (6 or 8)

Inserts aligned for VRU mounting hardware

Reference photo

Boom mounts — blue arrows = M6 Ruthex insert locations

Step 9: Bolt VRU to boom mount (REQUIRED)

After installing RX-M6Sx6.8 inserts, bolt the VRU to the boom mount using 2× M6×20 cap screws into the Ruthex inserts (blue arrows on boom mount faces).

Applies to configuration:

single-sheave

dual-sheave

Procedure:

Requires boom-08 Ruthex inserts installed first

Mount VRU on LH or RH boom mount depending on your configuration

Reference legend: Blue: M6×20 cap screw — VRU to boom mount (×2) · into RX-M6Sx6.8

Hardware

| Qty | Part / specification |
| --- | --- |
| 2 | M6 × 20 mm Socket Head Cap Screw — qty 2 — Into ruthex® RX-M6Sx6.8 inserts on boom mount |

Checks before continuing:

VRU seated against boom mount

Both M6×20 screws threaded into RX-M6Sx6.8 inserts

VRU secure — no movement at mount interface

Reference photo

Boom mount face — 2 blue arrows for VRU mounting screws

Step 10: Install LR970B boom IMU mount (REQUIRED)

Mount the LR970B boom IMU bracket to the boom lattice. The bracket fits in multiple lattice openings — choose the location that works best for your build. Install 2× ruthex® RX-M6Sx6.8 inserts, then secure with 2× M6×30 cap screws.

Applies to configuration:

single-sheave

dual-sheave

Procedure:

Mount location is flexible — bracket fits within multiple lattice areas

Install RX-M6Sx6.8 heat-set inserts at the two mounting holes (blue arrows) before bolting

Use 8.0 mm holes for RX-M6Sx6.8 inserts

Reference legend: Blue: M6×30 cap screw + RX-M6Sx6.8 insert (×2)

Printed parts

| Part name | STL file | Qty | Notes |
| --- | --- | --- | --- |
| Boom LR970B — Front Plate | BOOM LR970B FRONT PLATE.stl | 1 |  |
| Boom LR970B — Back Plate | BOOM LR970B BACK PLATE.stl | 1 |  |

Hardware

| Qty | Part / specification |
| --- | --- |
| 2 | ruthex® RX-M6Sx6.8 Threaded Insert — P/N GE-M6X68-001 — qty 2 — Heat-set at LR970B mounting holes on lattice |
| 2 | M6 × 30 mm Socket Head Cap Screw — qty 2 — Through LR970B mount flange into inserts |

Checks before continuing:

2× RX-M6Sx6.8 inserts installed flush at chosen mount location

LR970B front and back plates assembled on bracket

Bracket secured with 2× M6×30 cap screws

IMU mount solid — no flex at lattice interface

Reference photo

LR970B mount in lattice opening — blue arrows = 2 screw/insert locations

Boom Spacers for Dual Sheave

Segment status: Coming soon

STL library folder: BOOM SPACERS FOR DUAL SHEAVE

Content coming soon — add STL listing and assembly steps.

Boom Supports

Segment status: Coming soon

STL library folder: BOOM SUPPORTS

Content coming soon — stays, braces, and support hardware.

Multi Turn Add On

Segment status: Coming soon

STL library folder: MULTI TURN ADD ON

Content coming soon — winch, spool, and multi-turn adapter steps.

Bucket Attachment

Segment status: In progress

STL library folder: CLAMSHELL

Optional clamshell bucket end-effector for the bench crane. Complete the main crane build before installing this attachment.

Print tips:

Orient the clamshell halves tipped back on their rear exterior surface — open interior faces up (see Step 1 reference image).

Optional: use PLA for interface/support contact layers and PETG for the main model so supports break away cleanly.

Segment hardware summary

| Qty | Part / specification |
| --- | --- |
| 1 | 5 mm × 150 mm Sheave Pin — qty 1 — Bucket pivot — same 5 mm × 150 mm pin as boom sheave pins |
| 1 | 5 mm ID × 14 mm OD × 5 mm Ball Bearing — qty 1 — Same 5 mm × 14 mm × 5 mm bearing as boom sheave assembly |
| 8 | ruthex® RX-M3×5.7 Threaded Insert — P/N GE-M3X5.7-001 — qty 8 — Heat-set into 4.0 mm holes on bucket printed parts |
| 19 | M3 × 8 mm Socket Head Cap Screw — qty 19 — Linkage and bucket assembly |
| 1 | M3 × 12 mm Steel Dowel Pin — qty 1 — M3 × 12 mm alignment pin |

Print checklist

| Part name | STL file | Qty | Notes |
| --- | --- | --- | --- |
| Clamshell Half 1 | CLAMSHELL HALF 1.stl | 1 | Recommended orientation — see Step 1 |
| Clamshell Half 2 | CLAMSHELL HALF 2.stl | 1 | Recommended orientation — see Step 1 |
| Added Clearance Bucket Retainer | ADDED CLEARANCE BUCKET RETAINER.stl | 1 |  |
| Hanger 1 | HANGER 1.stl | 1 |  |
| Hanger 2 | HANGER 2.stl | 1 |  |
| Keeper | KEEPER.stl | 1 |  |
| Linkage Arm | LINKAGE ARM.stl | 4 |  |
| Top Section | TOP SECTION.stl | 1 |  |
| 30 mm Pulley Half 1 | 30MM PULLEY (PRIMARY SHEAVE) HALF 1.stl | 1 |  |
| 30 mm Pulley Half 2 | 30MM PULLEY (PRIMARY SHEAVE) HALF 2.stl | 1 |  |

Step 1: Print all bucket parts — recommended orientation (REQUIRED)

Print every STL in the CLAMSHELL folder. Lay the clamshell halves tipped back on their rear exterior surface with the open interior facing up, as shown in the reference image.

Warnings

Optional multi-material: PLA interface layers + PETG main model helps supports snap off cleanly

Procedure:

CLAMSHELL HALF 1 & 2: rear exterior on the build plate, open side up — matches the recommended slicer layout

Hangers, keeper, linkage arms, top section, retainer, and pulley halves: print flat on the build plate

LINKAGE ARM.stl — print 4 copies

Checks before continuing:

All bucket attachment STLs printed

Bucket halves oriented per reference image

Supports removed — no loose interface material in pivot or pin bores

Reference photo

Upload slicer layout screenshot to Drive → bucketPrintOrientation in CRANE_DRIVE_IMAGES

Step 2: Install ruthex® RX-M3×5.7 heat-set inserts (REQUIRED)

Install 8× ruthex® RX-M3×5.7 inserts (P/N GE-M3X5.7-001) in the bucket printed parts before assembly. Use 4.0 mm holes and the M3 ruthex tip — same inserts as the rest of the crane build.

Procedure:

Heat-set all inserts flush before mating linkage parts

Do not exceed 300°C when installing inserts

Hardware

| Qty | Part / specification |
| --- | --- |
| 8 | ruthex® RX-M3×5.7 Threaded Insert — P/N GE-M3X5.7-001 — qty 8 — 4.0 mm holes · GE-M3X5.7-001 |

Checks before continuing:

All 8× RX-M3×5.7 inserts installed flush

Threads clean — M3×8 cap screws start by hand

Reference photo

Insert locations on bucket linkage parts — photo coming soon

Step 3: Assemble bucket linkage & mount (REQUIRED)

Assemble the bucket linkage using the printed parts, 19× M3×8 cap screws, 1× M3×12 pin, 1× 5 mm × 150 mm pin, and 1× 5 mm × 14 mm × 5 mm bearing. Detailed step-by-step photos are being added.

Procedure:

Press the 5 mm × 14 mm × 5 mm bearing into the pivot bore before installing the 5 mm × 150 mm pin

Use the M3 × 12 mm pin where the linkage calls for a fixed alignment pin

Tighten all M3×8 cap screws into RX-M3×5.7 inserts — do not overtighten printed walls

Printed parts

| Part name | STL file | Qty | Notes |
| --- | --- | --- | --- |
| Clamshell Half 1 | CLAMSHELL HALF 1.stl | 1 |  |
| Clamshell Half 2 | CLAMSHELL HALF 2.stl | 1 |  |
| Added Clearance Bucket Retainer | ADDED CLEARANCE BUCKET RETAINER.stl | 1 |  |
| Hanger 1 | HANGER 1.stl | 1 |  |
| Hanger 2 | HANGER 2.stl | 1 |  |
| Keeper | KEEPER.stl | 1 |  |
| Linkage Arm | LINKAGE ARM.stl | 4 |  |
| Top Section | TOP SECTION.stl | 1 |  |
| 30 mm Pulley Half 1 | 30MM PULLEY (PRIMARY SHEAVE) HALF 1.stl | 1 |  |
| 30 mm Pulley Half 2 | 30MM PULLEY (PRIMARY SHEAVE) HALF 2.stl | 1 |  |

Hardware

| Qty | Part / specification |
| --- | --- |
| 19 | M3 × 8 mm Socket Head Cap Screw — qty 19 — Into RX-M3×5.7 inserts where noted · direct thread into pulley halves where noted. |
| 1 | M3 × 12 mm Steel Dowel Pin — qty 1 — Green arrows in step images · press-fit for alignment and joint support. |
| 1 | 5 mm × 150 mm Sheave Pin — qty 1 |
| 1 | 5 mm ID × 14 mm OD × 5 mm Ball Bearing — qty 1 |

Checks before continuing:

Bucket halves joined and linkage moves without binding

5 mm × 150 mm pivot pin installed with bearing seated

All 19× M3×8 cap screws tight

Bucket attachment ready to mount to the crane

Reference photo

Bucket assembly — step photos coming soon

Document control

Segments marked “Coming soon” or “In progress” reflect the current state of the interactive guide. Regenerate this document after guide updates to distribute the latest instructions.

Distribution: Trimble authorized personnel only. Do not post publicly or share outside Trimble without approval.

CONFIDENTIAL — Internal assembly documentation only.